200万定焦一体化云台枪机   2MP Fixed Lens Integration bullet PTZ Camera

DM-SLB405IAP-C04

- National invention patent design : The bullet camera with round bracket .(patent No : 2017300368380 )

- National invention patent design : The camera of round bracket . (patent No :2017300368376 )

- National invention patent design : The new gear reversing transmission technology . (patent No : 201710112514X )

- National invention patent design : Bullet PTZ Camera ( Patent No : 2017201860042 )

- National invention patent design : Integrated PTZ Camera structure . (Patent No : 2017201867431 )

- Built in RTC for Timing function .

- 2 MP 1/2.8”SC2135 CMOS Sensor + MSTAR MC313E DSP

- Support with 4mm fixed lens (Optional 4mm,6mm,12mm)

- Support with ONVIF .

- Pan speed : 0.1°~20°/ s ,tilt speed :  0.1°~12°/ s

- Double ICR , WDR ,ROI , BLC

- IP66 ,TVS 4000V Protection .

- APP Name : Danale

- Application scenario :

- Application for the place which are required more HD Image ,more beautiful ,such as warehouse , area , wharf ,square ,school ,park ,station .

Size dimensions :

Specs :

| Model |  |  | DM-SLB405IAP-C04 |
| --- | --- | --- | --- |
| camera | Audio | sensor | 1/2.8" CMOS |
|  |  | Image | 1080P(1920X1080) |
|  |  | shutter | 1/25s ~ 1/100000s |
|  |  | Iris | F2.0 |
|  |  | Day&Night | ICR |
|  |  | Illumination | 0.01Lux @(F2.0,AGC ON) ,0 Lux with IR |
|  | Video | Video compression | G.711-u law |
|  | Image enhance | Image setting | AWB, AGC adjust |
|  |  |  | Exposure Mode Manual /auto |
|  |  |  | Adjust , Sharpness, Saturation, Brightness & Contrast Adjustable ;white balance , |
|  |  | 3D Position | not support |
|  |  | DWR | D-WDR |
|  |  | D-Noise | 2D/3D-DNR |
|  |  | Image Orientation | Mirror/Flip |
|  |  | Privacy Zone | support |
|  |  | motion detection | support |
|  |  | OSD | Chinese &English |
| compression Standard |  | Chip | MSTAR MC313E |
|  |  | Video compression | H.264/ H265 |
|  |  | Frame | 50HZ:25fps(1920x1080) 60HZ:30fps(1920x1080) |
|  |  | Bit Rate | 32kbps-.3.5Mbps |
|  |  | rate control mode | VBR/CBR |
|  |  | image effect | VBR：5 adjustable mode CBR： Can adjustable freely |
| Cover/Housing |  | Focus | 6mm |
|  |  | Iris | Fixed Iris |
|  |  | lens connector | M12 connector |
|  |  | protection | IP66 |
|  |  | Bracket | wall bracket ,pend bracket |
|  |  | size | 168mm×68mm×68mm（length ,wide ,high |
|  |  | Gross | 360g |
| PTZ |  | Optical zoom | Fixed Focus: 6mm |
|  |  | IR Distance | 50M |
|  |  | Pan speed range | 0°~200° |
|  |  | Tilt speed range | 0°~60° |
|  |  | Pan speed | 0.1°~20°/ s |
|  |  | Tilt speed | 0.1°~12°/ s |
|  |  | Preset | Not support |
| communication interface |  | network | RJ45 |
|  |  | Power | DC 12V |
|  |  | T TF Card | （not support ） |
|  |  | Cloud storage | Not support |
|  |  | Audio | one Audio in ,one audio out ( optional) |
|  |  | Alarm | not supprot |
| network |  | Remote control | IE Browse |
|  |  | Phone client | Support with Windows Mobile、iPhone、iPad、Android |
|  |  | User | Supports simultaneous monitoring for up to 4 users with multi-stream real time |
|  |  | network protocol | TCP/IP, UDP, DHCP, NTP, RTSP, PPPoE, DDNS, SMTP, FTP |
|  |  | ONVIF | Support |
|  |  | Ethernet | 100Mbps |
|  |  | Wifi | not support |
|  |  | PoE | not support |
| others |  | Power | DC12V |
|  |  | Work Environment | -30℃~60℃ ,humidity range : 10%~90% |